There is a Desktrac RSS that programs the control parameters and can program the receiver and transmitter radios in a limited way - i.e. frequency and PL. 

For more extensive programming of the transceivers or for alignment - including setting the transmit power - you simply use the regular Maxtrac RSS. 

There is a programming selector switch on the front of the Desktrac - behind the little hole in the center of the unit. Position 1 accesses the transmitter, position 2 selects the receiver, and position 3 selects the control board. 

You use a regular Maxtrac programming cable plugged into the Desktrac's front panel mic jack.