Could someone tell me what the best connections would be to use a 
Motorola DeskTrac with the RC-210 for repeater control?

The following is the pinout of the Accessory # 2 Connector on a 
Desktrac:
1: GND - ground
2: N.C.
3: B+ - 13.8v dc, 750ma
4: PTT TEL - Active low level keys the TX with audio from pin # 5
5: MIC TEL - TX pre-emphasis audio input
6: GND - ground
7: DRXA TEL - Filtered & deemphasized RX audio output
8: GND - ground
9: Channel Monitor - Carrier squelch indicator output, active low
10: B+ - 13.8v dc, 750ma
11: N.C.
12: PL CH Monitor - Code squelch indicator output, active low
13: PTT - Active low level keys TX with flat TX audio
14: LINE PTT - PTT output from line control board
15: DRXA EXT LOCAL - Filtered & deemphasized RX audio
16: TXA EXT LCL - TX pre-emphasis audio input
17: TX CTCSS - TX CTCSS input
18: MONITOR - Monitor momentary input to unsquelch RX
19: PTT EXT LOCAL - Active low keys th TX with audio from pin # 16
20: N.C.
21: TX PL INHIBIT - TX PL inhibit input
22: TXA - TX flat audio input
23: RXA - RX flat (discriminator) audio output
24: PTT PAGER - Active low level keys TX audio from pin # 25
25: MIC PAGER - TX pre-emphasis audio input

This is what I thought:

Desktrac to RC-210 RJ-45
Pin #17 Pin # 1
Pin # 12 Pin # 2
Pin # 4 Pin # 3
Pin # 22 Pin # 4
Pin # 23 Pin # 5
Pin # 6 Pin # 6
Pin # 9 Pin # 7
Pin # 1 Pin # 8

Also should I change any of the jumper settings on the RC-210?

Thanks for all the help in advanced for a non-advanced person. Rod

Re: [rc210] Desktrac to RC-210 Help

Your Pin 17 (TX CTCSS) to Pin 1 isn't going to do anything. The RC210's CTCSS Encode Control output is not a tone output but rather a logic level only, designed to control an external encoder.

Other than that, you should be ok (I notice you're using discriminator audio and non-preemphasized xmt audio. Hopefully that flat audio input is limited correctly. Otherwise, you're going to make some adjacent channel enemies :-)

Ken

Re: [rc210] Desktrac to RC-210 Help

At 01:44 PM 10/31/06, you wrote:
>Your Pin 17 (TX CTCSS) to Pin 1 isn't going to do anything. The
>RC210's CTCSS Encode Control output is not a tone output but rather
>a logic level only, designed to control an external encoder.

Which can be accessed on Desktrac pin 21.

>Other than that, you should be ok (I notice you're using
>discriminator audio and non-preemphasized xmt audio. Hopefully that
>flat audio input is limited correctly. Otherwise, you're going to
>make some adjacent channel enemies :-)
>
>Ken

And if he wants to switch the receiver from carrier squelch
to PL squelch he can connect one of the RC210s logic
level output to pin 18 on the Desktrac.
Not the best way to do it, but it works...

Mike WA6ILQ